TN210_14.DOC

SYSOP REMOTE PARAMETERS (12-16)

***********************************

Parameter 12, TXDELAY

---------------------

Node  radio  TXD  is adjustable in 10 ms increments.     The value selected

should  be  based  both  on the node's T/R capability and on the ability of the

worst case  radio in the link to respond.    Factors  influencing  response and

approximate times are:

1.  2 - 20 ms to overcome receive squelch (if squelched).

2.  2 - 5 ms for switched crystal oscillators to settle.

3.  10 - 50 ms for older mechanical T/R relays to switch.

4.  5 - 10 ms for modern reed relays to switch.

5.  350 ms for PLL circuits to switch and settle.

Other than recommending users to upgrade their radios,  a LAN NodeOp has little

control over user radio TXD's.    This being the case,  the suggested TXD for a

LAN node is "30" (300 ms).  If certain  users experience repeated retries, than

TXD  should be bumped higher, as necessary.    Radios selected for backbone use

should be faster responding and TXDs can be set accordingly.

(Range:  0-255)

Parameter 13, BROADCAST VIA PORT

--------------------------------

Under certain networking configurations,  it's desirable to control whether

the  NODES broadcast will be allowed or not, over the RS-232 and the HDLC radio

ports.   A typical example would be an isolated LAN node where NODES broadcasts

over the radio port would just add QRM to the channel, since there are no other

nodes to receive them.   Another example would be an access node not wanting to

advertise  its  presence  onto a backboned trunking system.    In this case the

broadcast  would  be blocked  via the RS-232 port.   Even though broadcasts are

inhibited,  it  doesn't  prevent  reception  of  broadcasts  from  other nodes.

0  =  broadcasts via NO ports, 1  =  broadcasts via HDLC  radio port only,  2 =

broadcasts via RS-232 port only,  3 = broadcasts via both ports.

(Range: 0-3)

Parameter 14, # PROPAGATE

-------------------------

A portion of the  node-to-node  overhead is taken up with  the inclusion of

# hidden nodes in the NODES broadcasts.   In large backboned trunking networks,

the # hidden nodes can contribute significantly to network clutter.   Since the

status of the  # hidden nodes are  typically of interest only to local NodeOps,

it  would be desirable to limit their propagation into the system.    Suggested

value is "0."   0 = OFF, no propagation,  1 = ON, allowed to propagate.

(Range: 0-1)

Parameter 15, CONNECT COMMAND ENABLE

------------------------------------

A  factor  enhancing  improved  throughput on backboned trunking systems is

the  lack of collisions  due to  allowing  (in the most modern of systems) only

ONE transmitter  per  link frequency.     This configuration  can be maintained

simply  by disallowing  connected  users  to issue a  CONNECT  command FROM the

protected node.   User  connect  attempts  will be quietly ignored.   Transport

layer 4 connect  requests from adjacent nodes are not affected and are serviced

in the normal way.  With exception of the special case mentioned, the suggested

value is "1".    0 = CONNECT command disabled,  1 = CONNECT command enabled.

(Range: 0-1)

Parameter 16, DESTINATION LIST LENGTH

-------------------------------------

Limits  the maximum  number of nodes,  both hidden and non-hidden, that will

accumulate in the ROUTING table.   This table contains an  alphabetical listing

by  alias  of  distant  (destination)  nodes  picked up during neighbor node(s)

broadcasts.     This is the  same listing a  user sees  in response to a  NODES

command.    The term  "Destination"  implies  a user  is able to  make a single

connect to any  of the nodes listed in the 'destination table'.  With each node

listed,  the amount of "free buffers" within the node's memory space is reduced

by 36 bytes.    Depending  upon the  number of  nodes in the table,  the node's

memory  could  become  temporarily  depleted.  Should this happen, the response

to  certain  commands  will be   "Node busy".     The number of free buffers is

shown in parenthesis during the USERS command response.

Free  buffer depletion is not normally a problem with the typical node when the

maximum  table  size  is  limited to 100.   If the  Minimum Quality for Update,

parameter  1,  is  sized to  limit  the number  of  nodes  to  those  that  are

"connectable",  the  NODES  table will  normally contain far fewer than the 100

limit.  Suggested value is "100".

(Range: 1-400)